INFN — Sezione di Napoli

POEMMA-Balloon with Radar (PBR) — Clock Board

Software Design Document

Processing System application software — ethCmdPBR

| Field | Value |
| --- | --- |
| Document reference | PBR-CLKB-SDD-PS-001 |
| Issue / Revision | 1 / 0 |
| Date | 2026-07-10 |
| Standard | ECSS-E-ST-40C — Design Definition File (DDF) |
| Prepared by | M. Mese (INFN Napoli) |
| Status | Draft — generated from the as-built source code |

# Document Change Record

| Issue | Rev | Date | Description |
| --- | --- | --- | --- |
| 1 | 0 | 2026-07-10 | First issue. Reverse-engineered from the as-built source code. |

# Table of Contents

Document Change Record	2

Table of Contents	2

1  Introduction	4

1.1  Purpose and scope	4

Note on the origin of this document	4

1.2  Definitions, acronyms and abbreviations	4

2  Applicable and reference documents	4

3  Software design overview	5

3.1  Static architecture	5

3.2  Dynamic architecture	5

3.2.1  Concurrency and shared resources	5

3.3  Behaviour	6

3.3.1  Initialisation sequence	6

3.3.2  Acquisition loop	6

3.3.3  GPS handling	7

3.3.4  IMU handling	7

3.4  Interface context	7

3.4.1  UIO devices	7

3.4.2  Register map	7

3.4.3  TCP command interface	8

3.5  Memory and CPU budget	8

4  Software design (components)	9

4.1  main	9

4.2  registers	9

4.3  dma	9

4.4  acquisition	9

4.5  commands	10

4.6  tcpserver	10

4.7  gps	10

4.8  imu	10

4.9  crc32	11

5  Internal interface design	12

5.1  On-disk event record (pbrData_t)	12

5.2  Status register decoding	12

5.3  TCP command set	12

6  Requirements traceability	13

Annex A  Design observations	13

# 1  Introduction

## 1.1  Purpose and scope

This Software Design Document (SDD) describes the design of ethCmdPBR, the application software that runs under Linux on the Processing System (PS) of the Zynq-7000 device of the PBR Clock Board. The application configures and commands the Programmable Logic (PL) firmware, receives the event records produced by the PL through an AXI DMA channel, merges them with the GPS strings and the IMU attitude data, and writes the resulting records to disk for retrieval over FTP. It also exposes a TCP command and telemetry interface.

The scope of this document is limited to the PS application software. The PL firmware is described in PBR-CLKB-SDD-PL-001.

## Note on the origin of this document

This document has been produced by direct inspection of the as-built C sources of the ethCmdPBR repository. It describes the design as implemented, not a design intent captured before implementation. Sections that require information not contained in the source code — notably the software requirements baseline, the verification status, and the resource budgets — are explicitly marked as 'to be completed' rather than being populated with assumed values.

## 1.2  Definitions, acronyms and abbreviations

| Term | Definition |
| --- | --- |
| CRC | Cyclic Redundancy Check (IEEE 802.3, 32-bit) |
| DMA | Direct Memory Access |
| GTU | Gate Time Unit |
| IIO | Industrial I/O — the Linux subsystem used to read the IMU |
| IMU | Inertial Measurement Unit (Bosch BNO055) |
| NMEA / PTNL | GPS sentence formats; $PTNL marks the start of a Trimble sentence block |
| PL / PS | Programmable Logic / Processing System |
| PPS | Pulse Per Second |
| S2MM | Stream to Memory-Mapped (DMA write channel) |
| UIO | Userspace I/O — the Linux driver used to map the PL registers and receive interrupts |

# 2  Applicable and reference documents

| Ref | Item |
| --- | --- |
| [AD1] | ECSS-E-ST-40C — Space engineering: Software |
| [AD2] | ECSS-Q-ST-80C — Space product assurance: Software product assurance |
| [RD1] | PBR-CLKB-SDD-PL-001 — SDD, Programmable Logic firmware (clkBoardPBR) |
| [RD2] | Source repository ethCmdPBR (main.c, acquisition.c, commands.c, tcpserver.c, gps.c, imu.c, dma.c, registers.c, crc32.c) |
| [RD3] | Xilinx AXI DMA v7.1 Product Guide (PG021) |
| [RD4] | Bosch BNO055 datasheet; Linux IIO bno055 driver |

# 3  Software design overview

## 3.1  Static architecture

The application is a single Linux process, built by a plain Makefile (gcc, -Wall -Wextra -O2, linked against libpthread) into the executable ethCmd. It is decomposed into the following translation units.

| Module | Responsibility |
| --- | --- |
| main.c | Process entry point: opens and maps the UIO devices, initialises the DMA, creates the listening socket, starts the worker threads and then serves incoming TCP connections. |
| registers.c | UIO device lookup by name, memory-mapped register read/write helpers. |
| dma.c | AXI DMA S2MM driver: reset, interrupt enable, destination buffer programming, transfer and completion polling. |
| acquisition.c | checkFifoThread: the acquisition loop. Waits for the trigger interrupt, starts the DMA transfer, assembles the on-disk record, computes the CRC and writes the file. |
| commands.c | TCP command table, command lookup (sorted table + binary search), register write/read handlers, status register decoding. |
| tcpserver.c | Connection slot pool and per-connection command thread (cmdDecodeThread). |
| gps.c | gpsCfgIrqThread (fans the PL GPS-configuration interrupt out to the two GPS threads) and gpsCtrlThread (reads the serial sentences and applies the configuration string). |
| imu.c | IMU discovery through sysfs/IIO, scale metadata export, buffered acquisition thread, snapshot access. |
| crc32.c | Table-driven IEEE 802.3 CRC-32. |
| main.h | Shared configuration constants, thread argument structures and the on-disk record type pbrData_t. |

## 3.2  Dynamic architecture

The process runs the following threads. All threads are created during initialisation, except the per-connection command threads, which are created on demand and detached, and the IMU thread, which is started and stopped together with the acquisition run.

| Thread | Entry point | Trigger / blocking point | Purpose |
| --- | --- | --- | --- |
| main | main() | accept() | Accepts TCP connections and dispatches them to a slot of the connection pool. |
| acquisition | checkFifoThread | poll() on UIO 'trig' (100 ms timeout) | Per-event DMA transfer, record assembly and file writing; starts/stops the IMU thread following the run state. |
| GPS config fan-out | gpsCfgIrqThread | poll() on UIO 'gps_conf' | Re-arms the PL interrupt and notifies both GPS threads through eventfd. |
| GPS control (×2) | gpsCtrlThread | poll() on /dev/ttyUL{1,2} and eventfd | Accumulates a $PTNL sentence block into the shared GPS buffer; writes the configuration string on request. |
| IMU | imuThread | poll() on the IIO character device and stop eventfd | Copies the latest 56-byte scan into the shared snapshot. |
| command (×N, ≤ 5) | cmdDecodeThread | read() on the connection socket | Parses one command per line, executes it and returns the reply. |

### 3.2.1  Concurrency and shared resources

Two mutexes are used:

mtx — protects the memory-mapped PL registers, the shared GPS string buffer and the IMU snapshot. It is taken by the acquisition thread, by the GPS threads, by the IMU thread and by every command thread.

poolMtx — protects the inUse flags of the connection slot pool (acquireSlot / releaseSlot).

Locking discipline: imuGetSnapshot takes mtx internally, therefore the acquisition thread releases mtx before calling it, so that the two critical sections do not nest. The command threads hold mtx for the whole duration of decodeCmdStr, which serialises all register accesses against the acquisition thread. The number of simultaneous connections is bounded by CONN_MAX = 5; further connections are refused with an explicit message.

## 3.3  Behaviour

### 3.3.1  Initialisation sequence

main() performs, in order:

sortCmd(): sorts the command table so that it can be searched with bsearch.

Creation of one eventfd per GPS receiver, used to notify a configuration request.

Lookup and mmap of six UIO devices: AXIRegister@43c00000 (command register), AXIStatusReg@43c10000 (status and counters), AXIStatusReg@43c20000 (L1 counters), AXIStatusReg@43c30000 (PPS, alive/dead, trigger flags), dma@40400000 (AXI DMA) and dma_buffer (the DMA destination buffer).

DMA initialisation: reset, halt, enable all interrupts, then programming of the destination address DATA_ADDR = 0x1F000000.

Creation of the listening socket on TCP port 5000, with SO_REUSEADDR, and up to 10 retries on bind and listen.

IMU initialisation; if the device is absent the application continues without it.

Reading of the board identifier from the MAC address of interface end0 (low nibble of the last byte).

Creation of the acquisition thread, the GPS configuration thread and the two GPS control threads.

Entry into the accept() loop, which never returns.

### 3.3.2  Acquisition loop

checkFifoThread arms the PL interrupt by writing to the UIO file descriptor, then loops:

poll() with a 100 ms timeout on the 'trig' UIO device.

The status register is read under mtx; bit 0 (run) gives the current run state.

The IMU acquisition is started when the run starts and stopped when it stops.

On a trigger interrupt, dma_transfer_s2mm() moves the 24-byte PL record into the DMA buffer and polls the S2MM status until the IOC or IDLE flag is set (bounded by DMA_SYNC_MAX_RETRIES).

Every TRG_NUM_PER_FILE = 25 events a new output file is opened; the previous one is renamed to drop the .lock suffix, so that an FTP client only sees complete files.

The record is assembled, the CRC is computed over all fields except the CRC itself, and the record is appended to the current file.

The PL interrupt is re-armed.

When the run is not active, the event and file counters are reset and the current file is unlocked.

### 3.3.3  GPS handling

Each GPS receiver is reached through an AXI UART Lite exposed as /dev/ttyUL1 and /dev/ttyUL2, opened in canonical mode. A sentence block starts at each $PTNL token: when a new token is seen, the previously accumulated block is copied into the shared buffer under mtx, and a new block is started with the header "GPS<n>". A block that would exceed GPS_SLOT_LEN = 244 bytes is discarded. When the PL raises the configure_GPS interrupt, gpsCfgIrqThread re-arms the interrupt and signals both GPS threads, which write the 8-byte configuration string to their receiver.

### 3.3.4  IMU handling

The BNO055 is located by scanning /sys/bus/iio/devices for the device named bno055. The four scale factors (accelerometer, magnetometer, angular velocity, rotation) are read from sysfs and written, together with the record layout, to /srv/ftp/imu_metadata.txt, so that the raw counts stored in the event records can be converted offline. The acquisition uses the IIO buffered interface: the buffer is enabled at run start, a probe read validates the 56-byte scan size, and the thread then copies each scan into a shared snapshot protected by mtx. Each event record carries the most recent snapshot.

## 3.4  Interface context

### 3.4.1  UIO devices

| UIO name | Purpose | Interrupt |
| --- | --- | --- |
| AXIRegister@43c00000 | Command register (write) and FIFO status | — |
| AXIStatusReg@43c10000 | Status register, event and GTU counters, FIFO count | — |
| AXIStatusReg@43c20000 | L1 trigger counters (Zynq 0..3) | — |
| AXIStatusReg@43c30000 | PPS counter, alive/dead time, trigger flags | — |
| dma@40400000 | AXI DMA registers | — |
| dma_buffer | DMA destination buffer (physical 0x1F000000) | — |
| trig | Trigger event from the PL | IRQ_F2P[4] |
| gps_conf | GPS reconfiguration request from the PL | IRQ_F2P[5] |

openUioByName() resolves a UIO device by comparing the requested name against /sys/class/uio/uio*/maps/map0/name and /sys/class/uio/uio*/name.

### 3.4.2  Register map

| Symbol | Address | Access | Description |
| --- | --- | --- | --- |
| CMD_RECV_ADDR | 0x43C0_0000 | W | 32-bit command word to the PL command decoder. |
| FIFO_STATUS_ADDR | 0x43C0_0004 | R | FIFO status. |
| (status) | 0x43C1_0000 | R | Status register (see clause 5.2). |
| EVT_COUNTER_ADDR | 0x43C1_0004 | R | Event counter. |
| GTU_COUNTER_ADDR | 0x43C1_0008 | R | GTU counter. |
| FIFO_COUNTER_ADDR | 0x43C1_000C | R | AXI-Stream FIFO write data count. |
| L1_0..3_COUNTER_ADDR | 0x43C2_0000..0C | R | L1 trigger counters of Zynq 0..3. |
| PPS_COUNTER_ADDR | 0x43C3_0000 | R | PPS counter. |
| ALIVE_COUNTER_ADDR | 0x43C3_0004 | R | Live time (10 ns units). |
| DEAD_COUNTER_ADDR | 0x43C3_0008 | R | Dead time (10 ns units). |
| TRGFLG_COUNTER_ADDR | 0x43C3_000C | R | Trigger flags. |

writeReg() issues an msync(MS_SYNC) after each write. Register offsets are computed as (regAddr − baseAddr) >> 2 and applied to the mapped pointer.

### 3.4.3  TCP command interface

The server listens on TCP port 5000 and accepts at most CONN_MAX = 5 simultaneous connections. TCP keep-alive is enabled (idle 10 s, interval 5 s, 3 probes). On connection the server sends a banner of the form "CLK BOARD SN<xx>", where <xx> is the board identifier in hexadecimal. Commands are ASCII, terminated by CR or LF, at most CMD_MAX_LEN − 1 = 14 characters. Unknown commands return an error string. The command exit closes the connection.

## 3.5  Memory and CPU budget

To be completed. No measured figures (resident set size, per-thread CPU load, worst-case acquisition latency) are available from the source tree; they shall be established by measurement on the target and recorded here.

# 4  Software design (components)

## 4.1  main

| Field | Description |
| --- | --- |
| Type | C translation unit, process entry point. |
| Purpose | Initialise the platform resources and serve TCP connections. |
| Function | See the initialisation sequence in clause 3.3.1. After initialisation, main() loops on accept(), acquires a connection slot, sets the socket options and creates a detached command thread. |
| Subordinates | registers, dma, tcpserver, acquisition, gps, imu. |
| Dependencies | libpthread, Linux UIO, Linux sockets. |
| Interfaces | None external; owns axiRegisters_t and the thread argument structures. |
| Processing | Any failure during initialisation is fatal (exit(EXIT_FAILURE)), except the absence of the IMU, which is tolerated. |
| Data | boardID, the register mapping, the shared GPS buffer and the IMU snapshot are all owned by main()'s stack frame and shared with the threads by pointer. main() never returns, so their lifetime spans the whole process. |

## 4.2  registers

| Field | Description |
| --- | --- |
| Type | C module. |
| Purpose | Locate UIO devices and access the memory-mapped PL registers. |
| Function | openUioByName() globs /sys/class/uio/uio*, compares the map or device name and opens the corresponding /dev/uioN. readReg()/writeReg() apply a word offset to the mapped base pointer. |
| Subordinates | None. |
| Dependencies | glob, mmap, msync. |
| Interfaces | openUioByName, readReg, writeReg, axiRegisters_t. |
| Processing | AXI_MAP_SIZE = 64 KiB per mapping. |
| Data | None retained. |

## 4.3  dma

| Field | Description |
| --- | --- |
| Type | C module (bare-metal style register driver). |
| Purpose | Drive the AXI DMA S2MM channel. |
| Function | dma_init_s2mm() resets the engine, halts it and enables all interrupts. dma_set_buffer() writes the destination address. dma_transfer_s2mm() starts the engine, writes the transfer length and then calls dma_s2mm_sync(), which polls the status register until IOC or IDLE is set, aborting on internal, slave or decode errors, or after DMA_SYNC_MAX_RETRIES = 1 000 000 iterations. |
| Subordinates | None. |
| Dependencies | The DMA register block mapped by main(). |
| Interfaces | See dma.h. The S2MM register offsets follow PG021. |
| Processing | Completion is detected by polling, not by the DMA interrupt, even though the interrupt is enabled and routed to the PS. |
| Data | None retained. |

## 4.4  acquisition

| Field | Description |
| --- | --- |
| Type | C module, one thread. |
| Purpose | Produce the on-disk event records. |
| Function | See clause 3.3.2. genFileName() builds the file name /srv/ftp/clk<id>_event_<YYYYMMDDhhmmss>-<nnnn>.dat.lock; unlockFile() renames it by removing the trailing .lock so that only complete files are visible to the FTP client. |
| Subordinates | dma, crc32, imu. |
| Dependencies | mtx, the DMA buffer, the shared GPS buffer, the IMU snapshot. |
| Interfaces | checkFifoThread(void*). |
| Processing | boardID and the record header are read once at thread start (both are immutable after thread creation) and reused for every event. The header is DATA_HEADER = 0x424B4C43 ("CLKB" when written little-endian); for a non-zero board identifier the most significant byte is replaced by the ASCII digit of the identifier, yielding "CLK<n>". |
| Data | pbrData_t, see clause 5.1. |

## 4.5  commands

| Field | Description |
| --- | --- |
| Type | C module. |
| Purpose | Define and execute the TCP command set. |
| Function | A static table of cmd_t associates each command string with an opcode, a feedback string, a handler, a base address and a register address. sortCmd() sorts the table once at start-up; getCmd() locates a command with bsearch(). writeCmd() writes the opcode into the PL command register; readCmd() reads the requested register and formats the reply; decodeStatusReg() renders the status register as a human-readable string. |
| Subordinates | registers. |
| Dependencies | mtx (held by the caller). |
| Interfaces | sortCmd, decodeCmdStr. Command set in clause 5.3. |
| Processing | The status register decoding is parameterised by EXTTRG_NUM, PPS_NUM and ZYNQ_NUM, which must be kept equal to the VHDL generics extTrgNum, ppsNum and zynqNum. |
| Data | The command table. |

## 4.6  tcpserver

| Field | Description |
| --- | --- |
| Type | C module, one thread per connection. |
| Purpose | Manage the connection pool and the per-connection command loop. |
| Function | acquireSlot()/releaseSlot() implement a fixed pool of CONN_MAX slots guarded by poolMtx. cmdDecodeThread() sends the banner, then reads the socket, splits the input on CR/LF, and calls decodeCmdStr() for each complete line while holding mtx. |
| Subordinates | commands. |
| Dependencies | poolMtx, mtx. |
| Interfaces | acquireSlot, releaseSlot, cmdDecodeThread. |
| Processing | Threads are detached; the slot is released when the connection closes. |
| Data | cmdDecodeArgs_t pool. |

## 4.7  gps

| Field | Description |
| --- | --- |
| Type | C module, three threads (one fan-out, two control). |
| Purpose | Collect the GPS sentence blocks and apply the receiver configuration on request. |
| Function | See clause 3.3.3. |
| Subordinates | None. |
| Dependencies | mtx, the UIO 'gps_conf' device, the eventfd pair, termios. |
| Interfaces | gpsCfgIrqThread, gpsCtrlThread. |
| Processing | The configuration string gpsConfStr is an 8-byte binary sequence written verbatim to the receiver. |
| Data | Two GPS slots of GPS_SLOT_LEN = 244 bytes each, contiguous in the shared buffer. |

## 4.8  imu

| Field | Description |
| --- | --- |
| Type | C module, one thread. |
| Purpose | Provide the latest attitude sample to the acquisition thread. |
| Function | See clause 3.3.4. |
| Subordinates | None. |
| Dependencies | Linux IIO, sysfs, mtx, eventfd. |
| Interfaces | imuInit, imuStart, imuStop, imuThread, imuGetSnapshot. |
| Processing | imuRaw_t is a packed 56-byte structure; a _Static_assert enforces the layout against IMU_SCAN_SIZE. The scan layout is exported to /srv/ftp/imu_metadata.txt together with the scale factors. |
| Data | imuShared_t {latest, valid}. |

## 4.9  crc32

| Field | Description |
| --- | --- |
| Type | C module. |
| Purpose | Compute the record checksum. |
| Function | Table-driven IEEE 802.3 CRC-32, initialised with startCRC32 = 0xFFFFFFFF. |
| Subordinates | None. |
| Dependencies | None. |
| Interfaces | crc_32(data, dataLen, crc). |
| Processing | Applied to the first 576 bytes of pbrData_t, i.e. to every field except the crc field itself. |
| Data | Static lookup table. |

# 5  Internal interface design

## 5.1  On-disk event record (pbrData_t)

Each event is stored as one fixed-size record. The structure is not packed, but the only member with an alignment greater than 4 bytes (imuRaw_t) is declared packed, so the layout contains no padding. The measured size on the target toolchain is 580 bytes.

| Offset | Size | Field | Type | Description |
| --- | --- | --- | --- | --- |
| 0 | 4 | header | uint32 | 0x424B4C43 ("CLKB") or "CLK<n>" with the board identifier. |
| 4 | 4 | unixTime | uint32 | PS wall-clock time at record assembly (seconds since the epoch). |
| 8 | 4 | evtCount | uint32 | Event number (PL word 5). |
| 12 | 4 | gtuCount | uint32 | GTU since the last PPS (PL word 4). |
| 16 | 4 | trgFlag | uint32 | Trigger pattern (PL word 3). |
| 20 | 4 | aliveTime | uint32 | Live time in 10 ns units (PL word 2). |
| 24 | 4 | deadTime | uint32 | Dead time in 10 ns units (PL word 1). |
| 28 | 4 | status | uint32 | Status register (PL word 0). |
| 32 | 488 | gpsStr | char[488] | Two GPS slots of 244 bytes each. |
| 520 | 56 | imu | imuRaw_t | Latest IMU scan, packed, little-endian. |
| 576 | 4 | crc | uint32 | CRC-32 over bytes 0..575. |

Files are named /srv/ftp/clk<n>_event_<YYYYMMDDhhmmss>-<nnnn>.dat and contain TRG_NUM_PER_FILE = 25 records. While a file is being written it carries the additional suffix .lock.

## 5.2  Status register decoding

decodeStatusReg() renders the 32-bit status word as a sequence of "NAME=value" tokens. The bit positions are derived from the configuration macros, mirroring the concatenation performed in the PL (see PBR-CLKB-SDD-PL-001, clause 5.1).

/* fixed block, bits 10..0 */                /* parametric block, from bit 11 */
RUN_POS         = 0                          PARAM_BASE   = 11
RUNCTRLBUSY_POS = 1                          EXTTRG_POS   = PARAM_BASE
FIFOREADY_POS   = 2                          PPSEN_POS    = EXTTRG_POS  + EXTTRG_NUM
FIFOFULL_POS    = 3                          PPSPRES_POS  = PPSEN_POS   + PPS_NUM
BUSYCMD_POS     = 4                          ZQEN_POS     = PPSPRES_POS + PPS_NUM
PPSTRG_POS      = 5                          ZQBUSY_POS   = ZQEN_POS    + ZYNQ_NUM
GPSAUTO_POS     = 6
FSMSTATE_POS    = 7   (4 bits, 10..7)

The run-control state, extracted from bits 10..7, is rendered through the runCtrlDecode table as IDLE, STARTRUN, WAITTRG, TRGOR, TRGCPU, TRGEXT, TRGPPS, BUSYCPU, BUSYZYNQ, BUSY or ERR.

Configuration coupling: EXTTRG_NUM = 2, PPS_NUM = 3 and ZYNQ_NUM = 4 in commands.h must be kept consistent with the VHDL generics extTrgNum, ppsNum and zynqNum. This is a manual coupling and is not enforced by any build-time check.

## 5.3  TCP command set

| Command | Action |
| --- | --- |
| start run / stop run | Start or stop the acquisition run. |
| set busy / rel busy | Assert or release the Clock Board busy. |
| trg | Issue a software trigger. |
| gps configure | Request the reconfiguration of both GPS receivers. |
| gps1pps on¦no, gps2pps on¦no, clkpps on¦no | Enable or disable each of the three PPS sources. |
| ppsauto on¦no | Enable or disable the automatic PPS source selection. |
| ppstrg on¦off | Enable or disable triggering on the PPS edge. |
| msk exttrg0¦1, usk exttrg0¦1 | Mask or unmask each of the two external triggers. |
| zq0..zq3 on¦no | Enable or disable each front-end board. |
| gtu reset, evt reset, l1 reset, all reset | Reset the corresponding counters. |
| status | Return the decoded status register. |
| gtu counter, evt counter, pps counter | Return the corresponding counter. |
| l10..l13 counter | Return the L1 trigger counter of the given board. |
| help | List all commands with their description. |
| exit | Close the connection. |

# 6  Requirements traceability

To be completed, as for the PL document: no Software Requirements Specification has been identified in the project tree.

| Requirement ID | Component(s) | Verification method |
| --- | --- | --- |
| (to be defined) | (to be defined) | (to be defined) |

# Annex A  Design observations

The following points were identified during the inspection of the as-built code. They are recorded here because they affect the design description; none of them is a change request.

| # | Observation | Impact |
| --- | --- | --- |
| A.1 | In main(), the ioctl(fd, SIOCGIFHWADDR, …) that reads the board identifier is issued on the variable fd, which was closed after mapping the dma_buffer UIO device. The call succeeds only because the descriptor number has meanwhile been re-allocated to the listening socket by socket(). | Latent. Any change in the order of file-descriptor allocation would silently leave boardID at 0, i.e. every file would be named clk0_… and every record header would read "CLKB". Recommendation: pass listenfd explicitly. |
| A.2 | DATA_BYTES (= DATA_PL_BYTES + DATA_GPS_BYTES = 512) is defined in main.h but is not used; the record actually written is sizeof(pbrData_t) = 580 bytes. | Cosmetic. The macro is misleading and should be removed or corrected. |
| A.3 | EXTTRG_NUM, PPS_NUM and ZYNQ_NUM (commands.h) duplicate the VHDL generics extTrgNum, ppsNum and zynqNum. No build-time or run-time check verifies that they agree. | Configuration risk. A mismatch silently corrupts the decoding of the status register. Consider exposing the generics in a read-only PL register and checking them at start-up. |
| A.4 | The DMA completion interrupt (IRQ_F2P[2]) is enabled and routed to the PS, but the acquisition thread detects completion by polling the S2MM status register. | By design; recorded for completeness. |